EXPERIMENT - VI

TITLE : Write a program to control the color of the LED by turning 3 different

potentiometers. One will be read for the value of Red, one for the value of Green, and

one for the value of Blue.

NAME : Shinde Shubham Dnyandev,       CLASS : SY-B,            ROLL NO : 2317121.

CODE :

// C++ code

//

void setup()

{

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

int red_pot   = analogRead(A1);

int blue_pot  = analogRead(A2);

int green_pot = analogRead(A3);

int red_led   = map(red_pot, 0, 1023, 0, 255);

int blue_led  = map(blue_pot, 0, 1023, 0, 255);

int green_led = map(green_pot, 0, 1023, 0, 255);

analogWrite(11, red_led);

analogWrite(10, blue_led);

analogWrite(9, green_led);

}

CIRCUIT :